1. Adder Selection and Justification

For this project, two distinct 8-bit adder architectures were selected to explore the trade-offs between area efficiency and computational speed:

Selected Architectures:

- Ripple Carry Adder (RCA)

- Kogge-Stone Adder (KSA)

Justification:

- RCA (Baseline): The Ripple Carry Adder was chosen as the primary baseline. It represents the most area-efficient design with the simplest logic structure. It is ideal for testing the LLM's ability to generate basic structural Verilog and serves as a reference point for power and area optimization.

- KSA (Performance): The Kogge-Stone Adder was selected to represent high-performance parallel-prefix architectures. It is widely used in high-speed processors due to its logarithmic delay. This choice allows us to evaluate how well the LLM handles complex, multi-stage parallel logic and wide-span wiring.

Architectural Differences:

| Feature | Ripple Carry Adder (RCA) | Kogge-Stone Adder (KSA) |
| --- | --- | --- |
| Logic Depth | O(N) - Linear | O(log₂N) - Logarithmic |
| Complexity | Low (Simple FA cascade) | High (Complex prefix network) |
| Delay | High (Carry ripples through every bit) | Low (Parallel carry computation) |
| Area/Wiring | Minimal area and routing | Significant area and routing overhead |

2. Natural Language Descriptions

Below are the functional and structural descriptions used for the Verilog generation in Part 1.

Design 1: 8-Bit Ripple Carry Adder (RCA8)

This design implements an 8-bit adder using a ripple-carry architecture. It consists of eight Full Adder (FA) modules connected in a chain. The carry-out signal of the i-th bit serves as the carry-in for the (i+1)-th bit. The overall sum is calculated bit-by-bit sequentially. While this architecture minimizes the number of logic gates (Area), the critical path delay is proportional to the bit-width, as the carry must "ripple" from the least significant bit (LSB) to the most significant bit (MSB).

Design 2: 8-Bit Kogge-Stone Adder (KSA8)

This design implements an 8-bit Kogge-Stone parallel-prefix adder. The architecture is divided into three main stages:

- Pre-processing: Generation of initial Propagate (P) and Generate (G) signals for each bit.

- Carry-lookahead Network: A three-stage parallel prefix tree (with spans of 1, 2, and 4) that computes group carry signals in parallel.

- Post-processing: Final XOR operations between the propagate signals and the group carries to produce the Sum bits.

The KSA provides the fastest possible addition by reducing the carry propagation time to log_2(8) = 3 stages of logic, making it highly suitable for performance-critical applications despite its increased wiring density.